CAT (Centre of alternative technology) - water and energy management as of 2005.

Do not use high-energy appliances e.g. computer flat screens

The use of appliances are efficiently organized

Wood burners use locally resourced wood

As well as the 2-vault dry composting toilet, the new composting toilets use a leachfield - a soakaway for urine

Hydro power is turned off in summer (lack of water)

The main reservoir was a damned quarry pre-CAT

Private arrangement with the landowner - 100 yr. lease

Before CAT became so popular they used to be self-sufficient

The water usage is monitored now - each visitor is allowed 11 buckets each

Two header tanks (1500 ltrs) for the on-site visitor cabins are filled via a standpipe. The rest of the site draws directly from the main reservoir source as opposed to the cabins which are themselves sourced by a smaller pond fed from the main reservoir. The whole system is gravity fed.

Main reservoir

Pea gravel along bottom

Regular water testing

Sand on top

Biotic level above this - bacterial control

Ceramic filter		UV filter

Cottages

Restaurant

Showers

Cabins

Flushing toilets

1-2 lt / 3-5 lt

[The bowls have also been redesigned]

SEWAGE SYSTEM

Dry composting toilets

Grey/black water

Humanure - after 1 year

Cabins - 36 people	Main site

Flower beds and orchards

Primary solid settlement (septic) tank - 2m deep  	Septic tanks - 3m deep

Solids sink to bottom						Horizontal/vertical reed beds

Effluent				Zig-zag coppice-willow swale

Sludge is drained off

The siphon flushes when full		    River

Dosed off into 2 reed basins

The crust is nutrient rich

Blockages checked once/week

Air pipes increase aerobic breakdown		      Scooped off

Smells only during dosing

Inoculate new compost toilets

REED BEDS

6 vertical flow basins 1.5m deep

Gravel filled

Even spread of dose

Aerobic breakdown - air is trapped

Root exudates feed bacteria that feed off effluent

Surface will trap solids not captured earlier

3 vertical flow basins

Secondary solids collection tank

May form a crust	Hydroponic beds (nutrient rich)

Tomatoes and courgettes

Horizontal flow beds

Marsh marigolds/Watermint/Yellow flag iris/False bullrush

Pond under polytunnel

Duckweed controls algae		Groundwater

River

FLOW FORMS - figures of eight

Controlled	Modified

Oxygenation

Plant growth

Pond life

Solar roof  -	9500 kw/yr - £75,000 - 112 sq/m

INTERNAL GRID SYSTEM

Hydro power				Wind power

Storage batteries

Sold to regional electricity company

Immediate use

RAIN

Filter

Water butt - 1500 lt.

Height

Sprinkler pressure

PV CELLS

Overall efficiency			PV production (Peak output in mw/yr)

1978 - 8% 		1992 - 16% 		1976 = 1	1992 = 65

2$ per peak watt	5$ per peak watt

DRY COMPOST TOIILET

Green oak frame with wattle-and-daub in-fill

Roof clad in chestnut shingles

Wattling is of willow

Daub is of traditional mix of lime putty and cow dung

Compost chamber

Multi-layered vault

Fibrous OM 	Specially introduced decomposer organisms

WIND ENERGY

Poldaw 3.5m wind pump on show (1.8m and 5m also available)

Pumping depths up to 140m

Shallow wells

Daily output 80 cubic meters (18,000 gal)

FOOD STORE

Partially underground

Insulation

Refrigeration

5 degrees constant throughout year

Slate walls	Turf roof

Moderate outside temperature

Thermal mass - long time to heat or cool down

Bilberry turf									Gridwork of rope and sticks

Bracken

Cardboard

Polythene

Recycled cardboard										Corrugated iron

3-4" dia. forestry poles

8-9" dia. home grown timber